EXPERIMENT 1

CODING

clc;

clear;

close all;

% EXPERIMENT 1

% Design and Analysis of Butterworth IIR LPF and HPF

% Input Parameters

Fs = 1000;       % Sampling frequency (Hz)

Fp_LP = 100;     % LPF passband frequency (Hz)

Fst_LP = 150;    % LPF stopband frequency (Hz)

Fst_HP = 200;    % HPF stopband frequency (Hz)

Fp_HP = 250;     % HPF passband frequency (Hz)

Ap = 1;          % Passband attenuation (dB)

Ast = 40;        % Stopband attenuation (dB)

%% LOW-PASS FILTER DESIGN

Wp_LP = Fp_LP/(Fs/2);

Ws_LP = Fst_LP/(Fs/2);

% Calculate filter order

[N_LP, Wn_LP] = buttord(Wp_LP, Ws_LP, Ap, Ast);

% Design Butterworth Low-Pass Filter

[b_LP, a_LP] = butter(N_LP, Wn_LP, 'low');

% Display coefficients

disp('===== LOW-PASS FILTER =====');

disp(['Filter Order = ', num2str(N_LP)]);

disp('Numerator Coefficients (b):');

disp(b_LP);

disp('Denominator Coefficients (a):');

disp(a_LP);

% Frequency Response

[H_LP, f_LP] = freqz(b_LP, a_LP, 2048, Fs);

%% HIGH-PASS FILTER DESIGN

Wp_HP = Fp_HP/(Fs/2);

Ws_HP = Fst_HP/(Fs/2);

% Calculate filter order

[N_HP, Wn_HP] = buttord(Wp_HP, Ws_HP, Ap, Ast);

% Design Butterworth High-Pass Filter

[b_HP, a_HP] = butter(N_HP, Wn_HP, 'high');

% Display coefficients

disp('===== HIGH-PASS FILTER =====');

disp(['Filter Order = ', num2str(N_HP)]);

disp('Numerator Coefficients (b):');

disp(b_HP);

disp('Denominator Coefficients (a):');

disp(a_HP);

% Frequency Response

[H_HP, f_HP] = freqz(b_HP, a_HP, 2048, Fs);

%% MAGNITUDE RESPONSE

figure;

subplot(2,1,1);

plot(f_LP, 20*log10(abs(H_LP)+eps), 'b', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth Low-Pass Filter');

xline(Fp_LP, '--g', 'Passband Edge');

xline(Fst_LP, '--r', 'Stopband Edge');

ylim([-100 5]);

subplot(2,1,2);

plot(f_HP, 20*log10(abs(H_HP)+eps), 'r', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Butterworth High-Pass Filter');

xline(Fst_HP, '--r', 'Stopband Edge');

xline(Fp_HP, '--g', 'Passband Edge');

ylim([-100 5]);

%% PHASE RESPONSE

figure;

subplot(2,1,1);

plot(f_LP, unwrap(angle(H_LP))*180/pi, 'b', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('LPF Phase Response');

subplot(2,1,2);

plot(f_HP, unwrap(angle(H_HP))*180/pi, 'r', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('HPF Phase Response');

%% FILTER PERFORMANCE VERIFICATION

Mag_LP = 20*log10(abs(H_LP)+eps);

Mag_HP = 20*log10(abs(H_HP)+eps);

LP_Passband = Mag_LP(f_LP <= Fp_LP);

LP_Stopband = Mag_LP(f_LP >= Fst_LP);

HP_Stopband = Mag_HP(f_HP <= Fst_HP);

HP_Passband = Mag_HP(f_HP >= Fp_HP);

disp('===== PERFORMANCE VERIFICATION =====');

fprintf('LPF Passband Maximum Loss: %.2f dB\n', ...

-min(LP_Passband));

fprintf('LPF Stopband Minimum Attenuation: %.2f dB\n', ...

-max(LP_Stopband));

fprintf('HPF Stopband Minimum Attenuation: %.2f dB\n', ...

-max(HP_Stopband));

fprintf('HPF Passband Maximum Loss: %.2f dB\n', ...

-min(HP_Passband));

disp('Experiment Completed Successfully.');