EXPERIMENT 4

CODE

clc;

clear;

close all;

% EXPERIMENT 4

% Comparative Analysis of Butterworth,

% Chebyshev Type-I and Elliptic IIR Filters

%% Input Parameters

Fs = 1000;

Fp = 100;

Fst = 150;

Rp = 1;

Rs = 40;

%% Normalized Frequencies

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

%% Butterworth Filter

[Nb, Wnb] = buttord(Wp, Ws, Rp, Rs);

[bb, ab] = butter(Nb, Wnb, 'low');

%% Chebyshev Type-I Filter

[Nc, Wnc] = cheb1ord(Wp, Ws, Rp, Rs);

[bc, ac] = cheby1(Nc, Rp, Wnc, 'low');

%% Elliptic Filter

[Ne, Wne] = ellipord(Wp, Ws, Rp, Rs);

[be, ae] = ellip(Ne, Rp, Rs, Wne, 'low');

%% Display Filter Orders

disp('FILTER ORDER COMPARISON');

fprintf('Butterworth Order = %d\n', Nb);

fprintf('Chebyshev Type-I Order = %d\n', Nc);

fprintf('Elliptic Order = %d\n', Ne);

%% Display Coefficients

disp('BUTTERWORTH COEFFICIENTS');

disp('Numerator:');

disp(bb);

disp('Denominator:');

disp(ab);

disp('CHEBYSHEV TYPE-I COEFFICIENTS');

disp('Numerator:');

disp(bc);

disp('Denominator:');

disp(ac);

disp('ELLIPTIC FILTER COEFFICIENTS');

disp('Numerator:');

disp(be);

disp('Denominator:');

disp(ae);

%% Frequency Response

[Hb, f] = freqz(bb, ab, 2048, Fs);

Hc = freqz(bc, ac, 2048, Fs);

He = freqz(be, ae, 2048, Fs);

%% Magnitude Response Comparison

figure('Color','w');

plot(f, 20*log10(abs(Hb)+eps), ...

'b', 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(Hc)+eps), ...

'r--', 'LineWidth', 1.5);

plot(f, 20*log10(abs(He)+eps), ...

'k-.', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Experiment 4: IIR Filter Comparison');

legend('Butterworth', ...

'Chebyshev Type-I', ...

'Elliptic');

xline(Fp, ':g', 'Passband Edge');

xline(Fst, ':m', 'Stopband Edge');

xlim([0 Fs/2]);

ylim([-100 5]);

drawnow;

%% Phase Response

figure('Color','w');

plot(f, unwrap(angle(Hb))*180/pi, ...

'b', 'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(Hc))*180/pi, ...

'r--', 'LineWidth', 1.5);

plot(f, unwrap(angle(He))*180/pi, ...

'k-.', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('Experiment 4: Phase Response');

legend('Butterworth', ...

'Chebyshev Type-I', ...

'Elliptic');

drawnow;

%% Filter Order Bar Chart

figure('Color','w');

bar([Nb Nc Ne]);

set(gca, 'XTickLabel', ...

{'Butterworth','Chebyshev-I','Elliptic'});

ylabel('Filter Order');

title('Comparison of Filter Orders');

grid on;

drawnow;

disp('Experiment 4 Completed Successfully');