EXPERIMENT 5

CODE

clc;

clear;

close all;

% EXPERIMENT 5

% IIR Filter Realization Using

% Direct Form-I, Direct Form-II,

% Cascade and Parallel Structures

%% IIR Filter Coefficients

b = [0.0675 0.1349 0.0675];

a = [1 -1.1430 0.4128];

disp('IIR FILTER COEFFICIENTS');

disp('Numerator Coefficients:');

disp(b);

disp('Denominator Coefficients:');

disp(a);

%% Test Input: Unit Impulse

x = [1 zeros(1,99)];

%% Direct Form-I

y1 = filter(b, a, x);

%% Direct Form-II

% Direct Form-II transposed implementation

y2 = filter(b, a, x);

%% Cascade Realization

[sos, g] = tf2sos(b, a);

disp('CASCADE SECOND-ORDER SECTIONS');

disp(sos);

disp('Cascade Gain:');

disp(g);

y3 = g * x;

for k = 1:size(sos,1)

y3 = filter(sos(k,1:3), sos(k,4:6), y3);

end

%% Parallel Realization

[r, p, k] = residuez(b, a);

disp('PARALLEL REALIZATION');

disp('Residues:');

disp(r);

disp('Poles:');

disp(p);

disp('Direct Terms:');

disp(k);

% Reconstruct transfer function from partial fractions

[bp, ap] = residuez(r, p, k);

% Apply parallel equivalent transfer function

y4 = filter(bp, ap, x);

%% Compare Output Responses

figure('Color','w');

subplot(2,1,1);

stem(0:19, y1(1:20), 'b', 'filled');

hold on;

stem(0:19, y2(1:20), 'r--');

grid on;

xlabel('Sample Index');

ylabel('Amplitude');

title('Direct Form-I and Direct Form-II');

legend('Direct Form-I','Direct Form-II');

subplot(2,1,2);

stem(0:19, y1(1:20), 'b', 'filled');

hold on;

stem(0:19, y3(1:20), 'k--');

stem(0:19, y4(1:20), 'g:');

grid on;

xlabel('Sample Index');

ylabel('Amplitude');

title('Comparison of IIR Realization Structures');

legend('Direct Form-I', ...

'Cascade', 'Parallel');

drawnow;

%% Frequency Response

[H1, f] = freqz(b, a, 2048);

figure('Color','w');

plot(f/pi, 20*log10(abs(H1)+eps), ...

'b', 'LineWidth', 1.5);

grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)');

ylabel('Magnitude (dB)');

title('IIR Filter Frequency Response');

drawnow;

%% Numerical Comparison

fprintf('\nOUTPUT COMPARISON\n');

fprintf('Direct Form-I vs Direct Form-II Error = %.10g\n', ...

max(abs(y1-y2)));

fprintf('Direct Form-I vs Cascade Error = %.10g\n', ...

max(abs(y1-y3)));

fprintf('Direct Form-I vs Parallel Error = %.10g\n', ...

max(abs(y1-y4)));

disp('Experiment 5 Completed Successfully');