EXP 8

CODING

clc;

clear;

close all;

% EXPERIMENT 8

% FIR Filter Realization and Linear-Phase Analysis

%% 1. INPUT PARAMETERS

Fs = 1000;

Fp = 100;

Fst = 150;

N = 20;

Fc = (Fp + Fst)/2;

Wn = Fc/(Fs/2);

%% 2. FIR FILTER DESIGN

b = fir1(N, Wn, 'low', hamming(N+1));

a = 1;

disp('FIR FILTER COEFFICIENTS');

disp(b);

%% 3. DIRECT-FORM REALIZATION

x = [1 zeros(1,99)];

y_direct = filter(b,a,x);

%% 4. CASCADE REALIZATION

[sos,g] = tf2sos(b,a);

disp('CASCADE SECOND-ORDER SECTIONS');

disp(sos);

y_cascade = g*x;

for k = 1:size(sos,1)

y_cascade = filter(sos(k,1:3), ...

sos(k,4:6), y_cascade);

end

%% 5. LINEAR-PHASE ANALYSIS

[H,f] = freqz(b,a,2048,Fs);

phase_response = unwrap(angle(H))*180/pi;

%% 6. IMPULSE RESPONSE COMPARISON

figure('Color','w');

stem(0:29,y_direct(1:30),'filled');

hold on;

stem(0:29,y_cascade(1:30),'r--');

grid on;

xlabel('Sample Index');

ylabel('Amplitude');

title('FIR Direct and Cascade Realization');

legend('Direct Form','Cascade Realization');

drawnow;

%% 7. MAGNITUDE RESPONSE

figure('Color','w');

plot(f,20*log10(abs(H)+eps),'LineWidth',1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR Filter Magnitude Response');

xline(Fp,'--g','Passband Edge');

xline(Fst,'--r','Stopband Edge');

xlim([0 Fs/2]);

ylim([-100 5]);

drawnow;

%% 8. PHASE RESPONSE

figure('Color','w');

plot(f,phase_response,'LineWidth',1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('FIR Filter Phase Response');

drawnow;

%% 9. COEFFICIENT SYMMETRY CHECK

symmetry_error = max(abs(b-fliplr(b)));

fprintf('\nFIR FILTER ANALYSIS\n');

fprintf('Filter Order = %d\n',N);

fprintf('Symmetry Error = %.10f\n', ...

symmetry_error);

fprintf('Direct vs Cascade Error = %.10g\n', ...

max(abs(y_direct-y_cascade)));

if symmetry_error < 1e-10

disp('The FIR filter has symmetric coefficients.');

disp('The filter exhibits linear-phase characteristics.');

else

disp('The FIR coefficients are not perfectly symmetric.');

end

disp('Experiment 8 Completed Successfully.');