EXXPERIMENT 6

CODING

clc;

clear;

close all;

% EXPERIMENT 6

% Design and Analysis of FIR Digital Filters

% Using Fourier Series and Windowing Techniques

%% 1. INPUT PARAMETERS

Fs = 1000;       % Sampling frequency (Hz)

Fp = 100;        % Passband edge frequency (Hz)

Fst = 150;       % Stopband edge frequency (Hz)

Ap = 1;          % Passband attenuation (dB)

Ast = 40;        % Stopband attenuation (dB)

N = 20;          % FIR filter order

Fc = (Fp + Fst)/2;   % Cutoff frequency

fprintf('EXPERIMENT 6: FIR FILTER DESIGN\n');

fprintf('Sampling Frequency = %d Hz\n', Fs);

fprintf('Passband Frequency = %d Hz\n', Fp);

fprintf('Stopband Frequency = %d Hz\n', Fst);

fprintf('Filter Order = %d\n', N);

fprintf('Cutoff Frequency = %.2f Hz\n', Fc);

%% 2. NORMALIZED CUTOFF FREQUENCY

Wn = Fc/(Fs/2);

%% 3. WINDOW GENERATION

w_rect = rectwin(N+1);

w_hamming = hamming(N+1);

w_hanning = hann(N+1);

w_blackman = blackman(N+1);

%% 4. FIR FILTER DESIGN

b_rect = fir1(N, Wn, 'low', w_rect);

b_hamming = fir1(N, Wn, 'low', w_hamming);

b_hanning = fir1(N, Wn, 'low', w_hanning);

b_blackman = fir1(N, Wn, 'low', w_blackman);

%% 5. DISPLAY FILTER COEFFICIENTS

disp(' ');

disp('RECTANGULAR WINDOW COEFFICIENTS');

disp(b_rect);

disp('HAMMING WINDOW COEFFICIENTS');

disp(b_hamming);

disp('HANNING WINDOW COEFFICIENTS');

disp(b_hanning);

disp('BLACKMAN WINDOW COEFFICIENTS');

disp(b_blackman);

%% 6. FREQUENCY RESPONSE

[H1, f] = freqz(b_rect, 1, 2048, Fs);

H2 = freqz(b_hamming, 1, 2048, Fs);

H3 = freqz(b_hanning, 1, 2048, Fs);

H4 = freqz(b_blackman, 1, 2048, Fs);

%% 7. MAGNITUDE RESPONSE

figure('Color','w');

plot(f, 20*log10(abs(H1)+eps), ...

'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(H2)+eps), ...

'--', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H3)+eps), ...

'-.', 'LineWidth', 1.5);

plot(f, 20*log10(abs(H4)+eps), ...

':', 'LineWidth', 2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('FIR Low-Pass Filter Magnitude Response');

legend('Rectangular', 'Hamming', ...

'Hanning', 'Blackman', ...

'Location','southwest');

xline(Fp, '--', 'Passband Edge');

xline(Fst, '--', 'Stopband Edge');

xlim([0 Fs/2]);

ylim([-100 10]);

drawnow;

%% 8. PHASE RESPONSE

figure('Color','w');

plot(f, unwrap(angle(H1))*180/pi, ...

'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(H2))*180/pi, ...

'--', 'LineWidth', 1.5);

plot(f, unwrap(angle(H3))*180/pi, ...

'-.', 'LineWidth', 1.5);

plot(f, unwrap(angle(H4))*180/pi, ...

':', 'LineWidth', 2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (Degrees)');

title('FIR Low-Pass Filter Phase Response');

legend('Rectangular', 'Hamming', ...

'Hanning', 'Blackman', ...

'Location','best');

xlim([0 Fs/2]);

drawnow;

%% 9. WINDOW FUNCTION COMPARISON

figure('Color','w');

plot(0:N, w_rect, 'LineWidth', 1.5);

hold on;

plot(0:N, w_hamming, '--', 'LineWidth', 1.5);

plot(0:N, w_hanning, '-.', 'LineWidth', 1.5);

plot(0:N, w_blackman, ':', 'LineWidth', 2);

grid on;

xlabel('Sample Index');

ylabel('Amplitude');

title('Comparison of Window Functions');

legend('Rectangular', 'Hamming', ...

'Hanning', 'Blackman');

drawnow;

%% 10. FILTER PERFORMANCE ANALYSIS

H = {H1, H2, H3, H4};

names = {'Rectangular'; 'Hamming'; ...

'Hanning'; 'Blackman'};

fprintf('\nFILTER PERFORMANCE ANALYSIS\n');

fprintf('---------------------------------------------\n');

for k = 1:4

response = 20*log10(abs(H{k})+eps);

passband = response(f <= Fp);

stopband = response(f >= Fst);

passband_loss = -min(passband);

stopband_attenuation = -max(stopband);

fprintf('\n%s Window\n', names{k});

fprintf('Passband Maximum Loss = %.2f dB\n', ...

passband_loss);

fprintf('Stopband Minimum Attenuation = %.2f dB\n', ...

stopband_attenuation);

end

%% 11. FILTER ORDER COMPARISON

figure('Color','w');

bar([N N N N]);

set(gca, 'XTickLabel', ...

{'Rectangular','Hamming','Hanning','Blackman'});

ylabel('Filter Order');

title('FIR Filter Order Comparison');

grid on;

drawnow;

%% 12. RESULT

disp(' ');

disp('Experiment 6 Completed Successfully.');

disp('Four FIR window techniques were analyzed.');